Date

# Developing a Sky at night brightness predictor

# Mission Statement

Develop an autonomous solar-powered monitoring system that continuously measures sky brightness and atmospheric conditions to predict optimal stargazing opportunities while contributing to light pollution research.

Contents

Mission Statement	1

1.	Stakeholder analysis	2

Primary Stakeholder	2

Secondary Stakeholders	2

2.	system overview	2

2.1.	System Context Design:	2

2.2.	System Breakdown structure:	2

2.3.	Equipment list	4

3.	use cases	5

requirements	8

verification & Validation	8

Risk Register	8

Acceptance Test Plan	8

# Stakeholder analysis

## Primary Stakeholder

User (Gideon) - Astronomy enthusiast and systems engineer

- Wants: Reliable predictions for stargazing/astrophotography

- Needs: Low maintenance, accurate alerts, historical data

- Constraints: Budget <£150, DIY assembly capability

## Secondary Stakeholders

Citizen Science Community (Globe at Night, Dark Sky International)

- Wants: Standardized light pollution data

- Needs: Calibrated measurements, metadata, API access

Portfolio Reviewers (Future employers)

- Wants: Evidence of full-stack engineering capability

- Needs: Clean documentation, deployed system, measurable outcome.

# system overview

## System Context Design:

## System Breakdown structure:

    - Night sky brightness predictor:

      - Optical Sensing Unit

        - Camera Module (ESP32-CAM)

        - Light Sensor (TSL2591)

        - Optics & Weather Protection

      - Environmental Sensing Unit

        - Temperature Sensor (BME280)

        - Humidity Sensor (BME280)

        - Pressure Sensor (BME280)

    - Control & Processing Unit

      - Microcontroller (ESP32)

      - Local Storage (SD Card)

      - Communication Module (WiFi)

      - Firmware & Control Logic

    - Solar Panel (6V 5W)

      - Charge Controller (TP4056)

      - Battery Pack (2x 18650 Li-ion)

      - Power Regulation (Buck Converter)

      - Power Monitoring

    - Enclosure & Mounting

      - Weatherproof Housing (IP65)

      - Thermal Management

      - Mounting Hardware

      - Cable Glands & Sealing

    - DATA MANAGEMENT SUBSYSTEM

      - Data Ingestion Layer

        - MQTT Broker

        - Message Validator

        - Data Transformer

      - Storage Layer

        - Time-Series Database (InfluxDB)

        - Image Storage (File System)

        - Configuration Database

        - Backup System

      - Data Processing Pipeline

        - Image Analysis Engine

        - Statistical Aggregation

        - Data Quality Checks

    - ANALYSIS & PREDICTION SUBSYSTEM

      - Image Processing Module

        - Cloud Detection Algorithm

        - Star Visibility Estimation

        - Timelapse Generator

        - Anomaly Detection (Meteor/Satellite)

      - Machine Learning Engine

        - Feature Engineering Pipeline

        - Model Training System

        - Prediction Service

        - Model Performance Monitor

      - Analytics Module

        - Trend Analysis

        - Pattern Recognition

        - Comparative Analysis (vs historical)

    - USER INTERFACE SUBSYSTEM

      - Web Dashboard

        - Real-time Data Display

        - Historical Visualizations

        - Prediction View

        - System Health Monitor

        - Configuration Interface

      - API Layer

        - RESTful API

        - WebSocket (Real-time)

        - Authentication/Authorization

      - Notification System

        - Alert Generator

        - Push Notification Service

        - Email Notification Service

    - INTEGRATION & TEST SUBSYSTEM

      - System Integration

      - Calibration Procedures

      - Testing Framework

      - Documentation System

  - Equipment list:

| Component | Purpose | Est. Cost |
| --- | --- | --- |
| ESP32-CAM | Main controller with camera | £8-12 |
| TSL2591 light sensor | High-sensitivity lux measurement | £8-10 |
| BME280 sensor | Temperature, humidity, pressure | £5-8 |
| 6V 5W Solar Panel | Daytime charging | £12-18 |
| 18650 Battery Holder + 2x batteries | Power storage (3.7V Li-ion) | £10-15 |
| TP4056 charging module | Solar battery management | £5 |
| Weatherproof enclosure (IP65 rated) | Protect electronics | £15-25 |
| MicroSD card (32GB+) | Local image/data storage | £8-12 |
| Buck converter (3.3V/5V) | Power regulation | £4-6 |

# use cases

  - UC-001: Real-Time Sky Monitoring

Actor: System (Autonomous)
Precondition: System powered and connected
Trigger: Timer event (every 5 minutes)

Main Flow:

- System wakes from sleep mode

- Read TSL2591 light sensor → Sky brightness value

- Read BME280 → Temperature, humidity, pressure

- Timestamp all readings with RTC/NTP

- Store locally to SD card

- Transmit via MQTT to home server

- Home server validates and stores in InfluxDB

- Return to sleep mode

Success Criteria: Data successfully logged with <1% packet loss over 24h period

  - UC-002: Night Sky Image Capture

Actor: System (Autonomous)
Precondition: Night time detected (light level < threshold)
Trigger: Timer event (every 30 minutes during night)

Main Flow:

- System determines it is night (TSL2591 < 10 lux)

- Configure camera (ISO, exposure, focus)

- Capture RAW image

- Perform basic image validation (not corrupted)

- Save to SD card with metadata

- Generate thumbnail for transmission

- Send thumbnail + metadata via MQTT

- Full image retrieved during daily sync

Success Criteria: 95% of night hours captured successfully

  - UC-003: Manual Stargazing Quality Check

Actor: User (Gideon)
Precondition: User accessing web dashboard
Trigger: User navigates to current conditions page

Main Flow:

- User opens dashboard on mobile/desktop

- System displays:

  - Current sky brightness (lux + Bortle scale estimate)

  - Cloud coverage % (from last image analysis)

  - Temperature & humidity

  - "Stargazing Quality Score" (0-10)

  - Predicted score for next 6 hours

- User views latest sky image

- User decides whether to go outside

Success Criteria: Dashboard loads in <2 seconds, data <5 minutes old

  - UC-004: Automated Excellent Conditions Alert

Actor: System (Automated)
Precondition: User has enabled alerts
Trigger: Prediction model identifies high-quality viewing window

Main Flow:

- Prediction engine runs hourly forecast

- Score exceeds threshold (e.g., score ≥8/10)

- System checks viewing window duration (≥2 hours predicted)

- Alert generator creates notification

- Push notification sent to user's phone

- Email backup sent

- User receives alert: "Excellent stargazing predicted for next 3 hours!"

- User can tap notification → opens dashboard

Success Criteria: Notification delivered within 2 minutes of trigger, 90% positive predictive value

  - UC-005: Historical Trend Analysis

Actor: User (Gideon)
Precondition: >30 days of data collected
Trigger: User navigates to analytics page

Main Flow:

- User selects date range (e.g., last 3 months)

- System retrieves data from InfluxDB

- Generate visualizations:

  - Sky brightness over time (line chart)

  - Cloud coverage distribution (histogram)

  - Best viewing nights (calendar heatmap)

  - Light pollution trend (is it getting worse?)

- User exports data as CSV for external analysis

Success Criteria: Graphs render in <5 seconds for 3 months of data

  - UC-006: ML Model Retraining

Actor: System (Scheduled)
Precondition: >60 days of labeled data available
Trigger: Weekly scheduled job

Main Flow:

- Extract features from historical sensor data

- Generate labels (good/bad viewing nights based on actual conditions)

- Split data (80% train, 20% test)

- Train updated model (Random Forest)

- Evaluate performance on test set

- If accuracy > current model: deploy new model

- If accuracy < current model: log warning, keep old model

- Store model metrics for tracking

Success Criteria: Model accuracy ≥75%, false positive rate <20%

  - UC-007: Contribute to Citizen Science

Actor: User (Gideon)
Precondition: 6+ months of calibrated data
Trigger: User decides to contribute data

Main Flow:

- User navigates to "Export Data" page

- System generates standardized dataset:

  - Timestamps in UTC

  - Sky brightness in standard units (mag/arcsec²)

  - Location coordinates (obfuscated if desired)

  - Measurement methodology metadata

- User downloads CSV/JSON

- User uploads to Globe at Night or similar platform

- User's data contributes to light pollution maps

Success Criteria: Data format complies with Globe at Night specifications

  - UC-008: System Health Monitoring

Actor: System (Autonomous) & User
Precondition: System operational
Trigger: Continuous monitoring + user dashboard access

Main Flow:

- System continuously monitors:

  - Battery voltage (warn if <3.5V)

  - WiFi signal strength (warn if <-75dBm)

  - SD card free space (warn if <10%)

  - Last successful data transmission time

  - Sensor read error rate

- If anomaly detected → generate alert

- User views system health dashboard

- Dashboard shows:

  - Uptime %

  - Power generation vs consumption

  - Communication reliability

  - Sensor status (green/yellow/red)

Success Criteria: >98% uptime over 30 days (excluding intentional maintenance)

# requirements

# Functional

| Reqt ID | Statement |
| --- | --- |
| FR-001 | Data Acquisition |
| FR-001.1 | The system SHALL measure sky brightness in the range 0.01 - 100,000 lux |
| FR-001.2 | The system SHALL measure temperature in the range -20°C to +60°C with ±1°C accuracy |
| FR-001.3 | The system SHALL measure relative humidity 0-100% with ±3% accuracy |
| FR-001.4 | The system SHALL measure atmospheric pressure 300-1100 hPa with ±1 hPa accuracy |
| FR-001.5 | The system SHALL capture images with minimum resolution 640x480 pixels |
| FR-001.6 | The system SHALL timestamp all data with ±5 second accuracy (NTP synced) |
| FR-002 | Data Collection Frequency |
| FR-002.1 | The system SHALL sample light sensors every 5 minutes (±30 seconds) |
| FR-002.2 | The system SHALL sample environmental sensors every 15 minutes |
| FR-002.3 | The system SHALL capture images every 30 minutes during night hours (19:00-07:00) |
| FR-002.4 | Night hours SHALL be automatically adjusted based on sunset/sunrise times |
| FR-003 | Data Storage |
| FR-003.1 | The system SHALL store sensor data locally on SD card as backup |
| FR-003.2 | The system SHALL retain local data for minimum 7 days before overwrite |
| FR-003.3 | The system SHALL transmit data to home server within 10 minutes of collection |
| FR-003.4 | The home server SHALL store time-series data for minimum 1 year |
| FR-003.5 | The system SHALL store images for minimum 30 days |
| FR-004 | Image Analysis |
| FR-004.1 | The system SHALL detect cloud coverage as percentage 0-100% |
| FR-004.2 | The system SHALL classify sky as Clear (<20%), Partial (20-70%), Overcast (>70%) |
| FR-004.3 | The system SHALL estimate visible star count from images |
| FR-004.4 | The system SHALL generate timelapse videos from nightly image sequences |
| FR-005 | Prediction & Scoring |
| FR-005.1 | The system SHALL generate a Stargazing Quality Score (0-10 scale) |
| FR-005.2 | The system SHALL predict conditions for next 6 hours |
| FR-005.3 | The system SHALL update predictions every 1 hour |
| FR-005.4 | The system SHALL achieve minimum 75% accuracy on predictions (vs actual conditions) |
| FR-006 | User Notification |
| FR-006.1 | The system SHALL send alerts when predicted score ≥8/10 |
| FR-006.2 | The system SHALL deliver alerts within 5 minutes of trigger |
| FR-006.3 | The system SHALL support push notifications and email |
| FR-006.4 | Users SHALL be able to configure alert thresholds and quiet hours |
| FR-007 | User Interface |
| FR-007.1 | The system SHALL provide web dashboard accessible via standard browsers |
| FR-007.2 | The dashboard SHALL display real-time data with <5 minute latency |
| FR-007.3 | The dashboard SHALL render historical data for user-selected date ranges |
| FR-007.4 | The dashboard SHALL be mobile-responsive |
| FR-007.5 | The system SHALL provide RESTful API for programmatic access |
| FR-008 | System Management |
| FR-008.1 | The system SHALL log all errors and warnings |
| FR-008.2 | The system SHALL monitor and report battery voltage |
| FR-008.3 | The system SHALL monitor and report communication status |
| FR-008.4 | The system SHALL support remote firmware updates (OTA) |

# pERFORMANCE

| Reqt ID | Statement |
| --- | --- |
| PR-001 | Power Consumption |
| PR-001.1 | The outdoor unit SHALL operate continuously on 2x 18650 batteries (7.4V, 6000mAh total) for minimum 3 days without solar charging |
| PR-001.2 | Average power consumption SHALL NOT exceed 200mA @ 3.7V during normal operation |
| PR-001.3 | Sleep mode power consumption SHALL NOT exceed 10mA |
| PR-001.4 | Solar panel SHALL provide minimum 25Wh/day under typical UK winter conditions |
| PR-002 | Communication |
| PR-002.1 | Data transmission success rate SHALL be ≥95% over 24h period |
| PR-002.2 | Maximum packet size SHALL NOT exceed 4KB for sensor data, 50KB for image thumbnails |
| PR-002.3 | System SHALL tolerate WiFi outages ≤4 hours without data loss (local buffering) |
| PR-003 | Processing |
| PR-003.1 | Image analysis SHALL complete within 30 seconds of capture |
| PR-003.2 | ML prediction SHALL complete within 10 seconds |
| PR-003.3 | Dashboard page load time SHALL be <3 seconds on 10Mbps connection |
| PR-004 | Reliability |
| PR-004.1 | System uptime SHALL be ≥98% over 30-day period (excluding m |
| PR-004.2 | Mean time between failures (MTBF) SHALL be ≥180 days |
| PR-004.3 | Sensor read error rate SHALL be <1% under normal conditions |

# eNVIRONMENTAL

| Reqt ID | Statement |
| --- | --- |
| ER-001 | Operating Environment |
| ER-001.1 | Outdoor unit SHALL operate in temperature range -10°C to +50°C |
| ER-001.2 | Outdoor unit SHALL withstand relative humidity 0-100% |
| ER-001.3 | Enclosure SHALL provide minimum IP65 ingress protection |
| ER-001.4 | System SHALL withstand wind speeds up to 60 mph (96 km/h) |
| ER-001.5 | System SHALL operate in direct sunlight and rain simultaneously |
| ER-002: | Mounting |
| ER-002.1 | Total outdoor unit weight SHALL NOT exceed 1kg |
| ER-002.2 | Mounting system SHALL support installation on poles, walls, or roofs |
| ER-002.3 | Installation SHALL NOT require specialized tools beyond standard screwdrivers |

# iNTERFACE

| Reqt ID | Statement |
| --- | --- |
| IR-001 | Sensor Interfaces |
| IR-001.1 | TSL2591 SHALL connect via I2C at 400kHz |
| IR-001.2 | BME280 SHALL connect via I2C at 400kHz |
| IR-001.3 | Camera SHALL connect via dedicated ESP32-CAM interface |
| IR-001.4 | All I2C devices SHALL use unique addresses (no conflicts) |
| IR-002: | Communication Interfaces |
| IR-002.1 | ESP32 SHALL connect to WiFi 802.11 b/g/n (2.4GHz) |
| IR-002.2 | MQTT communication SHALL use protocol version 3.1.1 |
| IR-002.3 | API SHALL use HTTPS with TLS 1.2 minimum |
| IR-002.4 | WebSocket connections SHALL use WSS (secure) |
| IR-003 | Power Interfaces |
| IR-003.1 | Solar panel output SHALL be 6V ±10%, minimum 5W |
| IR-003.2 | Battery voltage SHALL be 7.4V nominal (2S Li-ion configuration) |
| IR-003.3 | Charge controller SHALL implement CC/CV charging profile |
| IR-003.4 | Over-discharge protection SHALL activate at 6.0V (3.0V per cell) |

# sAFETY & rEGULATORY

| Reqt ID | Statement |
| --- | --- |
| SR-001 | Electrical Safety |
| SR-001.1 | All outdoor wiring SHALL be rated for outdoor use |
| SR-001.2 | Li-ion batteries SHALL include overcurrent protection |
| SR-001.3 | System SHALL include reverse polarity protection |
| SR-001.4 | Enclosure SHALL prevent accidental contact with live circuits |
| SR-002 | Data Privacy |
| SR-002.1 | System SHALL NOT collect personally identifiable information |
| SR-002.2 | Images SHALL NOT intentionally capture neighboring properties |
| SR-002.3 | Location data SHALL be obfuscated to 1km precision for public sharing |
| SR-002.4 | API access SHALL require authentication |
| SR-003 | RF Compliance |
| SR-003.1 | WiFi transmitter SHALL comply with UK OFCOM regulations |
| SR-003.2 | ESP32 SHALL use certified module with CE marking |

# mAINTAINABILITY

| Reqt ID | Statement |
| --- | --- |
| MR-001 | Serviceability |
| MR-001.1 | Battery SHALL be user-replaceable without tools |
| MR-001.2 | SD card SHALL be accessible without disassembling weatherproof seal |
| MR-001.3 | Firmware updates SHALL be deployable remotely (OTA) |
| MR-001.4 | System SHALL support remote diagnostics via web interface |
| MR-002 | Calibration |
| MR-002.1 | Light sensor calibration SHALL be verifiable against known reference |
| MR-002.2 | System SHALL log calibration date and parameters |
| MR-002.3 | Recalibration interval SHALL NOT exceed 12 months |

# verification & Validation

| Requirement | Verification Method | Acceptance Criteria | Test Phase |
| --- | --- | --- | --- |
| FR-001.1 | Bench Test | Sensor reads 1, 100, 10000 lux correctly (±10%) | Unit Test |
| FR-001.2 | Bench Test | Compare to calibrated thermometer at 3 temperatures | Unit Test |
| FR-002.1 | Log Analysis | 288 readings per day (±5) over 7 day period | Integration |
| FR-003.3 | Log Analysis | 95% of packets arrive within 10 min over 24h | Integration |
| FR-004.1 | Image Analysis | Manual verification vs algorithm on 50 images | System Test |
| FR-005.4 | Field Test | Compare predictions vs actual over 30 nights | Validation |
| PR-001.1 | Power Test | Measure runtime on battery without solar | Unit Test |
| PR-004.1 | Field Test | Monitor uptime % over 30 days | Validation |
| ER-001.1 | Environmental Test | Freezer test (-10°C) and heat test (+50°C) | System Test |
| SR-002.3 | Code Review | Verify GPS coordinate rounding in export function | Unit Test |

# Risk Register

| Risk ID | Description | Probability | Impact | Mitigation |
| --- | --- | --- | --- | --- |
| R-001 | WiFi range insufficient at mounting location | Medium | High | Pre-test signal strength; Option to use ESP32 with external antenna or add WiFi repeater |
| R-002 | Condensation inside enclosure | High | Medium | Add desiccant packets; Use breather vents with membranes; Test in freezer→warm cycle |
| R-003 | Battery insufficient for cloudy periods | Medium | High | Oversize battery pack; Add low-power mode; Monitor battery voltage and alert |
| R-004 | Camera focus/exposure incorrect for night sky | Medium | Medium | Manual focus lock; Test multiple exposure settings; Add auto-adjustment algorithm |
| R-005 | Light pollution from nearby sources | Low | Medium | Site survey before installation; Use baffles/shielding; Document in metadata |
| R-006 | SD card corruption from power loss | Medium | Medium | Use industrial-grade SD card; Implement atomic writes; Regular backup to server |
| R-007 | Insufficient training data for ML model | Medium | Low | Start with rule-based algorithm; Collect 60+ days before ML; Label data manually initially |
| R-008 | Wildlife interference (birds, insects) | Low | Low | Smooth enclosure design; Regular cleaning schedule; Accept as limitation |